| Course Name: | Hardware Description Language Lab (2UXL401) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: | 2-2-2021 | Batch No: | B2 |
| Faculty Name: | Bhargavi maam | Roll No: | 1912052 |
| Faculty Sign & Date: |  | Grade/Marks: |  |

Experiment No: 2

Title: Study of  Multiplexer (Dataflow, Behavioral, Structural)

Aim and Objective of the Experiment:

Write a VHDL code for implementing a 4:1 multiplexer - Using when else construct ( Data Flow) - Using structural architecture Write a testbench to verify your results. Write a VHDL code for 16: 1 mux using 4:1 mux and gates. ( Structural) To study basic types of architectures and to understand the use of concurrent statements.

COs to be achieved:

CO 1: Use basic Concurrent and Sequential statements in VHDL and write codes for simple applications CO 2: Test a VHDL code and verify the circuit model. CO 3: Synthesize and Implement the designed circuits on CPLD/ FPGA.

Work to be uploaded

Code for 4:1 Mux , Code for 16:1 mux Test bench for 4:1 mux and 16:1 Mux and simulation waveform of the same. Scanned copy of post lab questions DataFlow library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity Mux_Dataflow_Vedant is port( i0,i1,i2,i3,s0,s1: In std_logic; y: Out std_logic ); end Mux_Dataflow_Vedant; architecture Mux_Dataflow_Vedant_arch of Mux_Dataflow_Vedant is signal a,b:std_logic; begin a<= not s1; b<=not s0; y<= (a and b and i0) or (a and s0 and i1) or (s1 and b and i2) or (s1 and s0 and i3) ; end Mux_Dataflow_Vedant_arch; Test Bench library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity Mux_Dataflow_Vedant_tb is end Mux_Dataflow_Vedant_tb; architecture Mux_Dataflow_Vedant_tb_arch of Mux_Dataflow_Vedant_tb is component Mux_Dataflow_Vedant is port( i0,i1,i2,i3,s0,s1: In std_logic; y: Out std_logic ); end component; signal i0,i1,i2,i3,s0,s1,y: std_logic; begin uut: Mux_Dataflow_Vedant port map( i0,i1,i2,i3,s0,s1,y); process begin i0<='0'; i1<='1'; i2<='1'; i3<='0'; s0<='0'; s1<='0'; wait for 10ns; s0<='0'; s1<='1'; wait for 10ns; s0<='1'; s1<='0'; wait for 10ns; s0<='1'; s1<='1'; wait for 10ns; end process; end; Behavioral Entity library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity Mux_Behavioural_Vedant is port( i: In std_logic_vector(3 downto 0); s: In std_logic_vector(1 downto 0); y: Out std_logic ); end Mux_Behavioural_Vedant; architecture Mux_Behavioural_Vedant_arch of Mux_Behavioural_Vedant is begin process(s,i) begin if(s="00") then y <= i(0); elsif(s="01") then y <= i(1); elsif(s="10") then y <= i(2); else y <= i(3); end if; end process; end Mux_Behavioural_Vedant_arch; Test Bench library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity Mux_Behavioural_Vedant_tb is end Mux_Behavioural_Vedant_tb; architecture Mux_Behavioural_Vedant_tb_arch of Mux_Behavioural_Vedant_tb is component Mux_Behavioural_Vedant is port( i: In std_logic_vector(3 downto 0); s: In std_logic_vector(1 downto 0); y: Out std_logic ); end component; signal i: std_logic_vector(3 downto 0); signal s: std_logic_vector(1 downto 0); signal y: std_logic; begin uut: Mux_Behavioural_Vedant port map(i,s,y); process begin i <= "1010"; s <= "00"; wait for 20ns; s <= "01"; wait for 20ns; s <= "10"; wait for 20ns; s <= "11"; wait for 20ns; end process; end;

Post Lab Subjective/Objective type Questions:

Upload Answer of following question before coming to next laboratory. Q1. Analyse the following code and explain its output. library ieee; use ieee.std_logic_1164.all; entity xyz is port(g1,g2,g3:in std_logic; sel: in std_logic_vector(2 downto 0); q:out std_logic_vector(7 downto 0) ); end xyz; architecture xyz_a of xyz is signal q_s: std_logic_vector(7 downto 0); Begin with sel select q_s<= "01111111" when "000", "10111111" when "001", "11011111" when "010", "11101111" when "011", "11110111" when "100", "11111011" when "101", "11111101" when "110", "11111110" when "111", "11111111" when others; q<=q_s when (g1 and not g2 and not g3)= '1' else "11111111"; end xyz_a_a; Q.2 Write a test bench for the above code.

Conclusion: We wrote a VHDL code for implementing a 4:1 multiplexer using structural and dataflow architecture Then we wrote a VHDL code for 16: 1 mux using 4:1 mux and gates

Signature of faculty in-charge with Date: