| Course Name: | Hardware Description Language Lab (2UXL401) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: | 23/03/2021 | Batch No: | B2 |
| Faculty Name: | BGK | Roll No: | 1912052 |
| Faculty Sign & Date: |  | Grade/Marks: |  |

Experiment No: 4

Title: Use of sequential statements:

- Counter using IC 74163

- Synchronous Counter using Flip flops(3 Bit)

Aim and Objective of the Experiment:

Write a VHDL code for implementing a JK flip flop. Write a VHDL code for implementing a 3bit asynchronous counter with use of JK flip flop component. ( Structural) Write a testbench to verify your results. Write a VHDL code for implementing a 4 bit up down synchronous counter using IC 74163 Write a testbench to verify your results. Also, generate a programming file and download the code on CPLD kit and verify the results for synchronous counter To study basic sequential statements of VHDL and to understand use of test bench for simulation.

COs to be achieved:

CO 1: Use basic Concurrent and Sequential statements in VHDL and write codes for simple applications CO 2: Test a VHDL code and verify the circuit model. CO 3: Synthesize and Implement the designed circuits on CPLD/ FPGA.

Work to be done

Upload codes for JK flip-flop, synchronous counter and synchronous counter using 74163. Also upload test benches and simulation for the same. ENTITY JK FF library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_arith.all; use ieee.std_logic_unsigned.all; entity jk_Vedant is port ( JK: IN STD_LOGIC_VECTOR(1 downto 0); clock: IN STD_LOGIC; reset: IN STD_LOGIC; q: out STD_LOGIC ); end jk_Vedant; architecture jk_Vedant_arch of jk_Vedant is signal q_s : std_logic := '0'; begin process(reset,clock) begin if (reset = '1')then q_s <='0'; elsif (clock'event and clock = '1')then case (JK) is when "00" => q_s <= q_s; when "01" => q_s <= '0'; when "10" => q_s <= '1'; when others => q_s <= not q_s; end case; end if; q <= q_s; end process; end jk_Vedant_arch; ENTITY Counter library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity async_Vedant is port( rst,clk:In std_logic; o:Out std_logic_vector(2 downto 0) ); end async_Vedant; architecture async_Vedant_arch of async_Vedant is component jk_Vedant is port( JK:In std_logic_vector(1 downto 0); reset:In std_logic; clock:In std_logic; q:Out std_logic ); end component; signal s:std_logic_vector(2 downto 0); begin asc0:jk_Vedant port map("11",rst,clk,s(0)); asc1:jk_Vedant port map("11",rst,s(0),s(1)); asc2:jk_Vedant port map("11",rst,s(1),s(2)); o<=s; end async_Vedant_arch; Testbench Counter library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use ieee.std_logic_arith.all; entity async_tb is end async_tb; architecture async_Vedant_arch of async_tb is component async_Vedant is port( rst,clk:In std_logic; o:Out std_logic_vector(2 downto 0) ); end component; signal rst,clk:std_logic; signal o:std_logic_vector(2 downto 0); begin uut:async_Vedant port map(rst,clk,o); process begin clk<='1'; wait for 10ns; clk<=not clk; wait for 10ns; end process; process begin rst<='1'; wait for 20ns; rst<='0'; wait for 200ns; end process; end async_Vedant_arch ; IC 74163 library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_arith.all; use ieee.std_logic_unsigned.all; entity IC74163_Vedant is port ( clk : in std_logic; sel : in std_logic_vector(3 downto 0); l : in std_logic_vector(3 downto 0); o : out std_logic_vector(3 downto 0) ); end IC74163_Vedant; architecture IC74163_Vedant_arch of IC74163_Vedant is signal s :std_logic_vector(3 downto 0); begin process(clk,sel,l) begin if (clk'event and clk='0') then if(sel(0) = '0') then s <= "0000"; elsif(sel(1) = '0') then s <= l; else if(sel = "1111") then s <= s+1; end if; end if; end if; end process; o <= s; end IC74163_Vedant_arch; Test Bench library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_arith.all; use ieee.std_logic_unsigned.all; entity IC74163_Vedant_tb is end IC74163_Vedant_tb; architecture IC74163_Vedant_tb_arch of IC74163_Vedant_tb is component IC74163_Vedant is port ( clk : in std_logic; sel : in std_logic_vector(3 downto 0); l : in std_logic_vector(3 downto 0); o : out std_logic_vector(3 downto 0) ); end component; signal clk: std_logic; signal sel, l, o: std_logic_vector(3 downto 0); begin IC74163: IC74163_Vedant port map(clk, sel, l, o); process begin clk <= '0'; wait for 20ns; clk <= '1'; wait for 20ns; end process; process begin sel <= "0000"; wait for 20ns; l <= "0000"; sel <= "1000"; wait for 20ns; sel <= "1111"; wait for 2000ns; end process; end IC74163_Vedant_tb_arch;

Post Lab Subjective/Objective type Questions:

Q1. If synchronous preset and asynchronous clear inputs are to be added in the above flip-flops then how will you modify the code? Q.2. What are the ways in which clock can be generated in test bench?

Conclusion: We wrote a VHDL code for implementing a 3bit asynchronous counter with use of JK flip flop component.

Signature of faculty in-charge with Date: