| Course Name: | Hardware Description Language Lab (2UXL401) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: |  | Batch No: |  |
| Faculty Name: |  | Roll No: |  |
| Faculty Sign & Date: |  | Grade/Marks: |  |

Experiment No: 6

Title: FSM implementation: Sequence detection

Aim and Objective of the Experiment:

Write a VHDL code for implementing a Moore type, non-overlapping sequence detector which detects “10101” sequence and gives output high. Write a test bench to verify your results. To study FSM implementation in VHDL and to understand use of test bench for simulation.

COs to be achieved:

CO 2: Test a VHDL code and verify the circuit model. CO 3: Synthesize and Implement the designed circuits on CPLD/ FPGA.

Work to be done

Upload VHDL codes for sequence generator FSM. Also upload test bench and simulation for the same.

Post Lab Subjective/Objective type Questions:

Upload Answer of following question before coming to next laboratory. Q1. What changes will you make in VHDL code if the same sequence generator is mealy overlapping type? Q2. Examine the following VHDL code and complete the following entity Problem Port (X, CLK : in bit; Z1, Z2 : out bit); end Problem; architecture Table of Problem is signal State, Nextstate: integer range 0 to 3 :=0; begin process(State, X) begin case State is when 0 => if X = „0‟ then Z1 <= „1‟; Z2 <= „0‟; Nextstate < = 0; else Z1 < =‟0‟; Z2 <=‟0‟;Nextstate < = 1; end if; when 1 => if X = „0‟ then Z1 <=„1‟; Z2 <= „1‟; Nextstate < = 1; else Z1 < =„0‟; Z2 <=‟1‟; Nextstate < = 2; end if; when 2 => if X = „0‟ then Z1 <=„0‟; Z2 <= „1‟; Nextstate < = 2; else Z1 < =„0‟; Z2 <=‟1‟; Nextstate < = 3; end if; when 3 => if X = „0‟ then Z1 <=„0‟; Z2 <= „0‟; Nextstate < = 0; else Z1 < =„1‟; Z2 <=‟0‟; Nextstate < = 1; end if; end case; end process; process(CLK) begin if CLK‟event and CLK =‟1‟ then State <= Nextstate; end if; end process; end Table; (a) Draw a block diagram of the circuit implemented by this code (b) Write the state table that is implemented by this code (c) Write the type of state machine

Conclusion:

Signature of faculty in-charge with Date:

| Course Name: | Hardware Description Language Lab (2UXL401) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: |  | Batch No: |  |
| Faculty Name: |  | Roll No: |  |
| Faculty Sign & Date: |  | Grade/Marks: |  |

Experiment No: 7

Title: FSM Implementation : Word Problem

Aim and Objective of the Experiment:

Word problem for development of state diagram and design using VHDL. To study FSM implementation in VHDL and to understand use of test bench for simulation.

COs to be achieved:

CO 1: Use basic Concurrent and Sequential statements in VHDL and write codes for simple applications CO 2: Test a VHDL code and verify the circuit model.

Work to be done

Draw FSM for given problem. Upload VHDL codes for FSM developed from word problem. Also upload test bench and simulation for the same.

Post Lab Subjective/Objective type Questions:

Upload Answer of following question before coming to next laboratory. # Q1. Write a VHDL code for FSM implementation of a following state diagram. # Write a test bench for the same. # Q2. Analyse the code and draw the state diagram library IEEE; use IEEE.STD_LOGIC_1164.ALL; use IEEE.STD_LOGIC_ARITH.ALL; use IEEE.STD_LOGIC_UNSIGNED.ALL; entity fsm is Port ( ip1 : in std_logic; ip2 : in std_logic; ip3 : in std_logic; op1 : out std_logic; op2 : out std_logic; op3 : out std_logic; reset : in std_logic; clk : in std_logic); end fsm; architecture fsm_a of fsm is type state_t is (s_idle,s1,s2,s3); signal present_state, next_state : state_t; process(present_state,ip1,ip2,ip3) begin case present_state is when s_idle => if (ip1 ='1') then op1<='1'; op2<='0'; op3<='0'; next_state<= s1; elsif (ip2='1') then op1<='0'; op2<='1'; op3<='0'; next_state<= s2; elsif (ip3='1') then op1<='0'; op2<='0'; op3<='1'; next_state<= s1; else op1<='0'; op2<='0'; op3<='0'; next_state<= s_idle; end if; when s1 => if (ip1 ='1') then op1<='1'; op2<='0'; op3<='0'; next_state<= s1; else op1<='0'; op2<='0'; op3<='0'; next_state<= s_idle; end if; when s2 => if (ip2 ='1') then op1<=‘0'; op2<=‘1';op3<='0'; next_state<= s2; else op1<='0'; op2<='0'; op3<='0'; next_state<= s_idle; end if; when s3 => if (ip3 ='1') then op1<='0'; op2<='0'; op3<='1'; next_state<= s3; else op1<='0'; op2<='0'; op3<='0'; next_state<= s_idle; end if; end case; end process; process(clk,reset) begin if reset = '1' then present_state <= s_idle; elsif clk'event and clk='1' then present_state<= next_state; end if; end process; end fsm_a;

Conclusion:

Signature of faculty in-charge with Date: